# DL Lab 3 Solution

Digital Logic (Universiti Teknologi Malaysia)

School of Computing Faculty of Engineering UNIVERSITI TEKNOLOGI MALAYSIA

| SUBJECT SESSION/SEM | : : | SECR1013 DIGITAL LOGIC …2…02…3…-2…0…24…/0…2……………………………………………. |  |
| --- | --- | --- | --- |
| LAB 3 | : | SYNCHRONOUS DIGITAL | COUNTER |
| NAME | : | …………………………………………………………………. |  |
| DATE | : | …11/06/2023 |  |

## Lab #3

Identifying the Properties of a Synchronous Counter

## Aims

  - Expose the student with experience on constructing synchronous counter circuit using Flip-Flop IC, Basic Gate ICs, Breadboard and ETS-5000 Digital Kit.

  - Promote critical thinking among students by analysing the given circuit and identifying the behaviour of the digital circuit.

## Objectives

The objectives of this lab activity are to:

  - Implement a synchronous counter circuit into physical circuit using Breadboard, Flip-Flops, Basic Gates and Switches.

  - Completing the next-state table of the counter circuit.

  - Sketch the state diagram of the counter circuit.

  - Identify the properties of the counter.

## Materials And Equipment

Materials and equipment required for this lab are as follows:

| Item Name | Number of Item |
| --- | --- |
| 1. Breadboard | 1 |
| 2. 7408 Quad 2-Input AND | 1 |
| 3. 7404 Hex Inverter | 1 |
| 4. 7432 Quad 2-input OR | 1 |
| 5. 7476 Dual J-K Flip Flop | 1 |
| 6. ETS-5000 Digital Kit | 1 |

## Preliminary Works

  - Determine the logic level for each input combinations in Table 1 so that the desired result can be realized.

## Table 1

| Desired Result | PRE’ | CLR’ | J | K | CLK | Q |
| --- | --- | --- | --- | --- | --- | --- |
| Set initial value Q = 1 | 0 | 1 | X | X | -- | 1 |
| Output Q stays the same | 1 | 1 | 0 | 0 | ⇓ | 1 |
| Output Q become 0, no change in asynchronous input | 1 | 1 | 1 | 1 | ⇓ | 0 |
| Output Q is not the previous Q | 1 | 1 | 1 | 1 | ⇓ | 1 |
| RESET Q | 1 | 1 | 0 | 1 | ⇓ | 0 |
| SET Q | 1 | 1 | 1 | 0 | ⇓ | 1 |

  - Answer all questions.

- Which state that JK flip-flop has, but not on SR flip-flop.

- Identify whether the JK flip flop in 7476, is a positive-edge triggered or negative- edge triggered flip flop.

## Lab Activities  (Design it using deeds software and complete)

- You are given a counter circuit as shown in Figure 4. the truth table

- Investigate the behaviour of the counter by observing the next state of the counter for all combination of Present State and X values. Complete the NextState table of the counter in Table 2. Ensure the Switch 0 is in HIGH state.

(0=LOW, 1=HIGH)

Table 2

| Switch 7 | Present State |  | Next State |  |
| --- | --- | --- | --- | --- |
| X | Q1 LED1 | Q0 LED 0 | Q1 LED1 | Q0 LED 0 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 |